Response to section 5.

Patent Title

5.1) Invention Description

Include i) its main components, systems and/or processes; and ii) the main vehicle systems (components, controllers, etc.) your invention operates with. Include any alternate configurations or methods.

The present invention provides a method and system for adaptive, predictive control of charging current based on temperature measurements obtained from the charge port interface module (CPIM) of electric vehicles to avoid unwanted overheating and prevent any adverse impact on charging performance.

5.2) Problem

Describe the problem that your invention solves or the need that it satisfies.

Electric vehicles (EVs) commonly support high-power charging through conductive charging interfaces, including alternating current (AC) and direct current (DC) charging systems. During high-current charging, electrical current passes through conductive pins and associated harnesses within a charge port interface module (CPIM). These conductive paths inherently generate heat due to resistive losses proportional to the square of the charging current.

Some modern electric vehicle supply equipment (EVSE) employ cooled charging handles or cables to mitigate thermal rise. However, due to limited thermal conductance between the external cooling system and internal vehicle-side components of the CPIM, such cooling measures may not effectively prevent localized temperature rise at the vehicle-side contact interface.

The CPIM typically includes one or more temperature sensors positioned proximate to current-carrying elements, such as DC positive and DC negative power pins and, in certain implementations, AC conductors. These temperature measurements may be used to monitor thermal conditions at the charging interface. Existing charging standards require that, upon detection of excessive temperature at the charging interface, the charging system reduce or terminate current to prevent thermal damage or safety hazards. Conventional approaches commonly rely on fixed temperature thresholds. When a threshold is exceeded, charging current is abruptly reduced to a predefined minimum level or terminated entirely. Termination or excessive reduction of current inevitably impact the charging performance of the vehicle, leading to slow charging, and in some cases failures that require a manual re-start of the charging session by the user.

5.3) Previous Solutions

Describe previously known solutions to the problem identified in §5.2. Consider the internal, competitor OEM, and supplier solutions known by all co-inventors. If the invention satisfies a new need, explain how the automotive industry previously functioned without it.

Existing solutions typically comprise of actively monitoring the temperature readings from the CPIM sensors and reactively derating current or terminating charging when the temperature exceeds safe temperature thresholds.

5.4) Previous Solution Drawbacks

Describe the drawbacks or deficiencies of the previous known solutions identified in §5.3. If the invention satisfies a new need, explain the opportunities the automotive industry may have missed.

a) Reactive Threshold-Based Derating

Existing systems often employ static temperature thresholds that trigger current reduction only after measured temperature exceeds a predefined value. Due to thermal inertia and sensor delay, temperature may overshoot the threshold before corrective action stabilizes the system. This can result in:

- Oscillatory current behavior,

- Abrupt current reductions,

- Reduced charging efficiency,

- Frequent fallback to minimum current or shutdown.

b) Inability to Adapt to Variable Contact Conditions

The effective electrical resistance and thermal dissipation characteristics of the CPIM can vary between charging sessions due to:

- Variations in mechanical mating quality,

- Surface oxidation or contamination,

- Connector wear or degradation,

- Ambient temperature differences,

- Differences among charging stations.

Conventional static derating strategies do not account for such variability. As a result, current limits must be conservatively calibrated to worst-case conditions, reducing achievable charging performance under normal conditions.

c) Limited Utilization of Multiple Temperature Sensors

Where multiple temperature sensors are present at different DC power pins of the CPIM, conventional systems may monitor only a maximum temperature value without distinguishing between symmetric bulk heating and localized contact degradation. Consequently, fault conditions and benign heating events may be treated identically, limiting diagnostic capability and response precision.

d) Late Detection of Thermal Instability

Reactive control approaches may attempt gradual current reduction while temperature continues to rise. Only after instability becomes apparent does the system revert to a predefined fallback current or shutdown state. This may delay protective action and lead to repeated negotiation with the external charger.

Accordingly, there exists a need for a charging control method that:

- Adapts to real-time thermal and electrical conditions,

- Utilizes multiple CPIM temperature sensors,

- Predicts future thermal behavior prior to exceeding safety limits,

- Maximizes allowable charging current while maintaining safety,

- Provides structured fallback behavior when stabilization is infeasible.

5.5) Invention Solution

Describe how your invention solves the problem identified in §5.2, even if you included this information in a previous subsection.

The present invention provides a method and system for adaptive, predictive control of charging current based on temperature measurements obtained from a charge port interface module (CPIM).

In one embodiment, the method uses at least two temperature sensors positioned near the respective DC power pins. A third temperature sensor associated with an AC conductor may optionally be used as an ambient temperature reference.

The proposed method includes the following key elements:

- Adaptive Thermal Model Identification

The system continuously estimates parameters of a discrete-time thermal model that relates CPIM temperature to charging current. The model incorporates:

- A thermal memory term representing heat dissipation dynamics,

- A heating term proportional to the square of charging current,

- A bias or ambient compensation term.

The parameters of this model are identified online using measured current and temperature data. By continuously updating model parameters, the system adapts to variations in:

- Effective contact resistance,

- Thermal resistance to ambient,

- Thermal capacity,

- Environmental conditions.

The proposed method enables estimation of separate models for each DC power pin, enabling independent characterization of localized heating behavior.

- Predictive Horizon-Based Current Limiting

Using the identified thermal model parameters, the system predicts future temperature evolution over a defined time horizon under candidate charging current values. The system determines a maximum allowable charging current that ensures predicted temperature does not exceed a predefined safety threshold within the prediction horizon.

This predictive current limit is calculated prior to exceeding temperature limits, thereby reducing overshoot and avoiding abrupt fallback conditions.

- Hybrid Dynamic Derating Structure

The predictive current limit is combined with a dynamic temperature-based derating controller. The commanded charging current is selected as a constrained value that satisfies:

- Dynamic temperature regulation,

- Predictive horizon constraints,

- System-level maximum limits.

This hybrid structure maintains compatibility with conventional threshold-based safety mechanisms while improving charging stability and performance.

- Multi-Sensor Asymmetry Detection

By independently estimating thermal parameters for multiple DC pins, the system can detect asymmetric heating behavior indicative of localized contact degradation or abnormal resistance. In such cases, the system may:

- Adjust predictive margins,

- Modify allowable current,

- Initiate fallback procedures.

- Structured Fallback Logic

If predicted temperature exceeds safety limits even at reduced current, or if measured temperature reaches a hard limit, the system transitions to a predefined fallback current level or terminates charging, consistent with applicable charging standards.

Detailed Derivation:

This section presents a possible derivation of the governing equations for the key elements of the proposed method described above.

| Nomenclature |  |
| --- | --- |
| Parameter | Definition |
| Tk+ | filtered temperature reading from the sensor near DC positive pin |
| Tk- | Filtered temperature reading from the sensor near DC negative pin |
| TDC+ | temperature reading from the sensor near DC positive pin |
| TDC- | temperature reading from the sensor near DC negative pin |
| TAC | temperature reading from the sensor near AC pin |
| Tsoft | Temperature threshold where derating begins (e.g., 50°C) |
| Tlim | hard safety limit (e.g., 60°C) |
| ∆ | headroom margin for prediction (e.g., 2–5°C) |
| H | look-ahead time (e.g., 10–30 s) |
| Imax | charger/cable/battery max allowable current |
| Ifallback | usually 0A |

A conservative governing temperature can be assumed from arbitrating the sensor readings:

TkDC=max⁡(Tk+, Tk-)

where Tk+ and Tk- are the filtered values of temperature readings from sensors located at the positive and negative DC pins respectively.

An adaptive Thermal Model can be identified for the CPIM. To achieve this, a standard first-order thermal model for the CPIM “hot spot” (the location a sensor represents) is developed:

CdT(t)dt  =  P(t)  -  T(t)-Tamb(t)Rθ
 where:

- T(t)= temperature at the sensor location (°C)

- Tamb(t)= effective ambient/baseline temperature seen by that location (°C)
(could be cabin/underhood air, nearby structure, etc.)

- C= effective thermal capacitance (J/°C) at that location

- Rθ= effective thermal resistance to ambient (°C/W)

- P(t)= heat input (W)

For Joule heating near the pin/contact, heating is proportional to the square of current:

P(t)≈I(t)2 Req

where Reqis an effective electrical resistance “mapped” to that sensor’s hot region (includes contact + nearby conductor effects).

By substituting P(t) and rearranging into standard linear form:

| dTdt+1τT=ReqCI(t)2+1τTamb(t) | (xx) |
| --- | --- |

where τ=RθC is the time constant.

To get an explicit solution of eq (xx), it can be assumed that the inputs are constant over a short interval tktk+1 of length Δt:

- I(t)≈Ik

- Tamb(t)≈Tamb,k

Therefore, within that interval eq (xx) becomes:

| dTdt+1τT=ReqCIk2+1τTamb,k | (xx) |
| --- | --- |

in which the right-hand side is constant.

Equation (xx) can be written as:

Tk+1=aTk+bIk2+c

where

a=e-Δt/τ
b=τ1e-Δt/τReqC

c=1e-Δt/τTamb,k.

The key parameters of the model can be estimated online using the Recursive Least Squares (RLS) approach. The regressor model can be defined as:

yk=Tk+1=ϕkTθ+ϵk
where ϕk=TksIk21 is the regressor and θs=asbscs is the parameter vector. The RLS update can be achieved using the following definition for the Kalman gain as

| Kk=Pk-1ϕkλ+ϕkTPk-1ϕk | (3-1) |
| --- | --- |

such that:

| θk=θk-1+KkykϕkTθk-1, | (3-2) |
| --- | --- |

with covariance matrix Pk defined as:

| Pk=1λPk-1KkϕkTPk-1 | (3-3) |
| --- | --- |

where λ is the forgetting factor.

Once the model is at hand, it can be used to

By defining a soft target below the safety limit as

Ttgt=Tsoft(or Tlim-Δ)

temperature error can be computed based on the governing DC temperature

ek=Ttgt-TkDC

A PI controller on temperature is used to define a desired current as follows before prediction limitations are accounted for:

IkPI=Ik-1PI+Kp(ek-ek-1)+Kiek

Limited by the physics of the system:

Ikdyn= clip(IkPI,0, Imax).

The goal of the look-ahead limiter is to compute the largest constant current I(or a cap) such that predicted max temperature over the next Hseconds stays below b Tpred,lim=Tlim-Δ. To achieve this, one possible approach is to try different candidates for current I and predict for each DC sensor using its identified model using the following steps:

- Let N=⌈H/Δt⌉.

- Initialize the temperature  Tk,0s=Tks

- Iterate j=0..N-1:  Tk,j+1s=asTk,js+bsI2+cs

- set peak of  Tmax⁡(I)=max⁡jTk,js

- find a conservative peak across the two DC pins: Tmax(I)=max⁡Tmax⁡(I), Tmax⁡(I)

- finally find Ikpred=max⁡{I∈[0,Imax]∣Tmax(I)≤Tpred,lim}.

The final current command will be the minimum of the predicted current Ikpred, dynamically adjusted current from the PI controller  Ikdyn and the maximum allowed current of the system Imax,sys:

| Ikcmd=min⁡(Ikdyn, Ikpred,Imax⁡,system) | Eq (xxx) |
| --- | --- |

Proper rate limiting will be applied to Ikcmd to ensure compliance with hardware limitations of the vehicle and the EVSE:

| Ikcmd= clip(Ik-1cmd+ΔI↓, Ikcmd, Ik-1cmd+ΔI↑) | Eq (xxx) |
| --- | --- |

The strategy will still include fallback to a minimum safe current (0) if the temperature continues to increase above Tlim. Similarly, if the prediction leads to a current that is less than a minimum allowed threshold Ikpred≤Imin, the algorithm will terminate charging.

5.6) Innovation/Differences

Describe how your invention’s solution to the problem is different than the other previously known solutions identified in §5.3, even if you included this information in a previous subsection. What sets your invention apart from the known solutions is what could be claimed in a patent application.

The present invention provides multiple technical advantages over conventional temperature-based charging control systems used in electric vehicle charge port interface modules (CPIMs).

(a) Improved Thermal Stability and Reduced Overshoot

Conventional systems typically rely on static temperature thresholds that trigger reactive current reduction once a measured temperature exceeds a predefined value. Due to thermal inertia and sensor delay, such approaches may result in temperature overshoot, oscillatory current behavior, and abrupt derating events.

In contrast, the disclosed system incorporates predictive modeling of CPIM thermal dynamics. By estimating thermal parameters in real time and predicting future temperature evolution over a defined horizon, the system determines an allowable charging current before temperature exceeds a safety threshold. This reduces overshoot, minimizes oscillations, and provides smoother current regulation.

(b) Adaptive Response to Variable Contact and Environmental Conditions

Existing charging control systems generally assume fixed thermal characteristics and therefore must be calibrated conservatively to accommodate worst-case contact resistance and cooling conditions.

The present invention adaptively estimates thermal and heating parameters during each charging session using measured current and temperature data. As a result:

- If contact resistance is low and thermal dissipation is favorable, higher charging current may be safely permitted.

- If contact resistance increases due to wear, contamination, or partial insertion, allowable current is automatically reduced.

- Variations in ambient temperature and cooling conditions are inherently accounted for.

This adaptive capability enables improved charging performance without compromising safety.

(c) Enhanced Utilization of Multiple Temperature Sensors

Where multiple CPIM temperature sensors are available, conventional systems typically monitor only the maximum temperature and apply uniform derating logic.

The present invention independently models thermal behavior associated with each DC power pin. This allows:

- Detection of asymmetric heating conditions,

- Identification of localized contact degradation,

- Differentiation between bulk heating and localized fault conditions.

Such multi-sensor identification improves diagnostic capability and enables more targeted mitigation strategies.

(d) Early Detection of Thermal Instability

Reactive derating strategies may gradually reduce current while temperature continues to rise, only resorting to hard shutdown once a limit is exceeded.

By predicting temperature trajectories under candidate current levels, the disclosed system can identify conditions in which no feasible current value will maintain thermal stability. In such cases, the system transitions promptly to a predefined fallback current or shutdown state, reducing prolonged exposure to unsafe thermal conditions.

(e) Reduced Dependence on Conservative Calibration

Because conventional systems cannot adapt to session-specific conditions, calibration must assume worst-case resistance and cooling scenarios. This can unnecessarily limit charging speed under normal operating conditions.

The adaptive predictive framework of the present invention reduces reliance on fixed conservative assumptions by continuously identifying actual thermal parameters during operation. This enables operation closer to true thermal limits while maintaining compliance with safety requirements.

(f) Compatibility with Existing Charging Standards

The disclosed method operates entirely through software-based current request management and does not require modification of charging hardware or communication standards. The system remains compatible with existing AC and DC charging protocols by:

- Issuing current requests within permitted ranges,

- Maintaining predefined hard temperature limits,

- Executing fallback current reduction or shutdown when required.

(g) Implementation Without Additional Hardware

The invention leverages temperature sensors already present within many CPIM designs. No additional cooling hardware, mechanical actuators, or structural modifications are required. The method may be implemented within existing vehicle charging control units via software or firmware updates.

Technical Effect

Collectively, the invention provides:

- Increased allowable charging current under favorable conditions,

- Reduced oscillatory behavior,

- Improved safety through predictive limitation,

- Enhanced fault discrimination capability,

- Improved user charging experience.

These advantages are achieved through adaptive identification of CPIM thermal characteristics combined with horizon-based predictive current limiting and multi-sensor analysis.